Below is a complete, copy-pasteable **implementation guide (.txt style)** for your project. It lists what files to create, the exact header interfaces to implement, the minimal `main.cpp` wiring, and a short technical appendix that explains the concepts you asked about.

# Project layout (what to add and what to modify)

Create:

* `include/sample.hpp`

* `include/concurrent_queue.hpp`

* `include/mcp2221.hpp`

* `include/icm20948.hpp`

* `include/fusion.hpp`

* `include/redis_logger.hpp`

* `include/config.hpp`

Modify:

* `mcp2221.cpp` → implement `mcp2221.hpp` using your existing code, add a mutex.

* `icm20948.cpp` → implement `icm20948.hpp`, move the FIFO loop into a private producer thread.

* `Madgwick.cpp` → leave as is; `fusion.hpp` wraps it.

* `main.cpp` → use the new startup sequence.

Build system:

* Add `-Iinclude` to include path.

* Link hiredis if you use Redis now, else stub `RedisLogger` and add it later.

──────────────────────────────────────────────────────────────────────────────

# include/sample.hpp

```cpp

#pragma once

#include <cstdint>

#include <chrono>

// One IMU sample ready for logging or UI.

struct ImuSample {

int imu_id;                                   // set by device ctor

uint64_t seq;                                 // per-IMU incrementing sequence

std::chrono::steady_clock::time_point t_host; // capture time on host

// Scaled sensor data (keep units consistent across app).

float ax, ay, az;   // accel  (m/s^2 or g)

float gx, gy, gz;   // gyro   (rad/s or dps)

// Minimal status flags.

uint8_t fifo_overflow:1;

uint8_t fifo_underflow:1;

uint8_t mag_ok:1;

uint8_t reserved:5;

};

```

# include/concurrent_queue.hpp

```cpp

#pragma once

#include <mutex>

#include <condition_variable>

#include <deque>

template <typename T>

class ConcurrentQueue {

public:

void enqueue(const T& v) {

{ std::lock_guard<std::mutex> lk(m_); q_.push_back(v); }

cv_.notify_one();

}

bool try_dequeue(T& out) {

std::lock_guard<std::mutex> lk(m_);

if (q_.empty()) return false;

out = q_.front(); q_.pop_front(); return true;

}

T wait_dequeue() {

std::unique_lock<std::mutex> lk(m_);

cv_.wait(lk, [&]{ return !q_.empty(); });

T v = q_.front(); q_.pop_front(); return v;

}

size_t size() const { std::lock_guard<std::mutex> lk(m_); return q_.size(); }

private:

mutable std::mutex m_;

std::condition_variable cv_;

std::deque<T> q_;

};

```

# include/mcp2221.hpp

```cpp

#pragma once

#include <cstdint>

#include <vector>

#include <mutex>

// Thread-safe facade over your existing MCP2221 implementation.

// Implement in mcp2221.cpp by delegating to your current code.

// Lock bus_m_ in every I2C function.

class MCP2221 {

public:

bool open();                 // open USB device

void close();                // close USB device

void wait(int ms);           // sleep helper (you already have it)

// I2C ops. Return codes match your current implementation.

int i2cWrite(uint8_t addr, const std::vector<uint8_t>& data);   // write [reg, payload...]

int i2cRead(uint8_t addr, uint8_t reg, std::vector<uint8_t>& out);  // read N bytes from reg

int i2cReadOnly(uint8_t addr, std::vector<uint8_t>& out);           // read N bytes from current pointer

uint8_t i2cReadSingle(uint8_t addr, uint8_t reg);                   // read 1 byte

int GetLastError() const;

private:

mutable std::mutex bus_m_;   // lock every public I2C method with this

// keep your existing handles here

};

```

# include/icm20948.hpp

```cpp

#pragma once

#include <cstdint>

#include <vector>

#include <atomic>

#include <thread>

#include "mcp2221.hpp"

#include "sample.hpp"

#include "concurrent_queue.hpp"

// Configuration structures

struct ImuRanges {

float accel_scale = 1.0f;  // LSB -> m/s^2 (or g)

float gyro_scale  = 1.0f;  // LSB -> rad/s (or dps)

};

struct ImuConfig {

int fifo_packet_size_bytes = 12;   // accel+gyro (no temp)

int fifo_packet_mult_burst = 10;   // packets per burst read (10..15 ok)

int fifo_underflow_threshold_packets = 2;

int fifo_overflow_threshold_packets  = 10;

int odr_hz = 225;                  // device sample rate

ImuRanges ranges{};

bool enable_mag = false;           // reserved for future

};

class ICM20948 {

public:

ICM20948(MCP2221& mcp, uint8_t i2c_addr, int imu_id);

bool Initialize();                     // WHOAMI, reset, enables, base config

bool FIFOConfig();                     // FIFO sources and mode

bool CalibrateAccelGyro(uint16_t N);   // fill gyro_bias_ and accel_bias_

void set_zero_euler(const float euler0[3]); // optional UI zeroing

// Start/stop the producer thread that polls FIFO and enqueues ImuSample.

void start(ConcurrentQueue<ImuSample>& out, const ImuConfig& cfg);

void stop();

int id() const { return id_; }

uint8_t address() const { return addr_; }

private:

// Thread body

void producer_loop_(ConcurrentQueue<ImuSample>* out, ImuConfig cfg);

// Low-level helpers (implement with your existing code)

bool select_bank_(uint8_t bank) const;

bool write_reg_(uint8_t reg, uint8_t val) const;

bool read_regs_(uint8_t reg, uint8_t* buf, size_t n) const;

static uint16_t merge_hl_(uint8_t hi, uint8_t lo) { return (uint16_t(hi) << 8) | lo; }

bool read_fifo_count_(uint16_t& count) const;                      // reads COUNTH/L

bool read_fifo_burst_(std::vector<uint8_t>& buf, size_t n) const;  // reads FIFO_RW n bytes

void parse_fifo_packet_(const uint8_t* pkt, const ImuConfig& cfg, ImuSample& s) const; // scale+bias

// Optional IMU-only fusion for preview (not used by logger)

void fuse_imu_only_(const ImuSample& s, float euler_deg[3]); // calls Madgwick IMU path

// State

MCP2221& mcp_;

uint8_t addr_;

int id_;

std::atomic<bool> run_{false};

std::thread th_;

float gyro_bias_[3]  = {0,0,0};

float accel_bias_[3] = {0,0,0};

float euler_zero_[3] = {0,0,0};

};

```

# include/fusion.hpp

```cpp

#pragma once

// Thin wrappers around your Madgwick.cpp functions.

// Implement these by forwarding to MadgwickAHRSupdateIMU and QuaternionsToEulerAngles.

struct FusionState {

// optional: store beta and quaternion if your Madgwick exposes them

};

void fusion_update_imu(FusionState& st,

float gx_rad_s, float gy_rad_s, float gz_rad_s,

float ax, float ay, float az,

float dt_seconds);      // call MadgwickAHRSupdateIMU

void fusion_quat_to_euler(float euler_rad[3]); // call QuaternionsToEulerAngles

```

# include/redis_logger.hpp

```cpp

#pragma once

#include <vector>

#include <thread>

#include <atomic>

#include "sample.hpp"

#include "concurrent_queue.hpp"

struct RedisConfig {

const char* host = "127.0.0.1";

int port = 6379;

int batch_size = 100;          // pipeline size

};

class RedisLogger {

public:

explicit RedisLogger(RedisConfig cfg) : cfg_(cfg) {}

void attach(ConcurrentQueue<ImuSample>* q) { queues_.push_back(q); }

void start();   // connect to Redis and spawn logger thread

void stop();    // signal and join

private:

void loop_();   // drain queues, batch, XADD imu:<id> * ts <ns> seq <n> ax ... gz ...

RedisConfig cfg_;

std::vector<ConcurrentQueue<ImuSample>*> queues_;

std::atomic<bool> run_{false};

std::thread th_;

// hiredis context pointer lives in the .cpp

};

```

# include/config.hpp

```cpp

#pragma once

#include <string>

#include "icm20948.hpp"

#include "redis_logger.hpp"

struct AppConfig {

ImuConfig imu_cfg;

RedisConfig redis;

};

// For now you can hardcode defaults in the .cpp and ignore 'path'.

AppConfig load_config(const std::string& path);

```

──────────────────────────────────────────────────────────────────────────────

# main.cpp minimal wiring (adjust to your style)

```cpp

#include "mcp2221.hpp"

#include "icm20948.hpp"

#include "concurrent_queue.hpp"

#include "redis_logger.hpp"

#include "config.hpp"

int main() {

AppConfig app = load_config("config.toml"); // or stub

MCP2221 bus;

if (!bus.open()) return -1;

ICM20948 imu1(bus, 0x69, 1);

ICM20948 imu2(bus, 0x68, 2);

if (!imu1.Initialize() || !imu2.Initialize()) return -1;

if (!imu1.FIFOConfig()  || !imu2.FIFOConfig()) return -1;

imu1.CalibrateAccelGyro(1000);

imu2.CalibrateAccelGyro(1000);

ConcurrentQueue<ImuSample> q1, q2;

imu1.start(q1, app.imu_cfg);   // producer thread #1

imu2.start(q2, app.imu_cfg);   // producer thread #2

RedisLogger logger(app.redis);

logger.attach(&q1);

logger.attach(&q2);

logger.start();

// TODO: UI loop or blocking wait; press a key to exit, etc.

logger.stop();

imu1.stop();

imu2.stop();

bus.close();

return 0;

}

```

──────────────────────────────────────────────────────────────────────────────

# What to implement in your existing .cpp files

mcp2221.cpp

* Implement all methods from `mcp2221.hpp`.

* Pattern for each I2C method:

* `std::lock_guard<std::mutex> lk(bus_m_);`

* Call your existing low-level function body.

* Keep `wait(ms)` and `GetLastError()` as you have them.

icm20948.cpp

* Map your existing helpers to the private methods declared in `icm20948.hpp`:

* `select_bank_`, `write_reg_`, `read_regs_`, `read_fifo_count_`, `read_fifo_burst_`, `parse_fifo_packet_`.

* Replace your public blocking `ReadFIFO()` with:

* `start(...)` → launches `producer_loop_(...)`.

* `producer_loop_`:

* Read `FIFO_COUNTH/L`.

* If below threshold, continue.

* Read a burst bytes from `FIFO_RW`.

* `auto t_read = std::chrono::steady_clock::now();`

* For each packet in the burst:

* `ImuSample s{ id_, ++seq, t_read, ... }`.

* `parse_fifo_packet_(pkt, cfg, s)`.

* `out->enqueue(s)`.

* Adjust burst size when over/under thresholds like you do today.

* Remove keyboard polling, file I/O, and heavy `std::cout` from device code.

Madgwick.cpp

* No change. `fusion.hpp` functions forward to it.

redis_logger.cpp

* Implement `start()` to create a thread that runs `loop_()`.

* `loop_()`:

* Connect to Redis once.

* While `run_`:

* Try `try_dequeue()` from each attached queue.

* Accumulate up to `batch_size` samples.

* Pipeline `XADD` commands.

* Flush pipeline and handle errors with simple backoff.

config.cpp

* Return sensible defaults:

* `imu_cfg.fifo_packet_size_bytes = 12;`

* `imu_cfg.fifo_packet_mult_burst = 10;`

* `imu_cfg.odr_hz = 225;`

* `imu_cfg.ranges.accel_scale = <your scale>;`

* `imu_cfg.ranges.gyro_scale  = <your scale>;`

* `redis.host = "127.0.0.1"; redis.port = 6379; redis.batch_size = 100;`

──────────────────────────────────────────────────────────────────────────────

# Minimal unit conversions you must keep consistent

* Choose one unit set and keep it everywhere.

* Option A: accel in **m/s²**, gyro in **rad/s**.

* Option B: accel in **g**, gyro in **dps**.

* Set `ImuRanges` accordingly when you parse FIFO packets. Document it once in a comment.

──────────────────────────────────────────────────────────────────────────────

# Technical appendix (short, concrete explanations)

**mutex (`std::mutex`)**

* A mutex is a lock. One thread at a time can hold it. Use it to protect shared resources (your USB/I²C handle).

* Pattern: `std::lock_guard<std::mutex> lk(m); /* safe critical section */`.

**mutable**

* `mutable` lets you modify a member even inside a `const` method.

* Here it lets `i2cRead(...) const` lock `bus_m_`. The method stays `const`, the lock still works.

**condition_variable**

* Lets a thread sleep until another thread signals “data arrived”.

* Used in `ConcurrentQueue::wait_dequeue()` to block efficiently until there is an item.

**atomic (`std::atomic<bool> run_`)**

* Read/write without data races between threads.

* Here it controls the lifetime of the producer and logger loops.

**producer–consumer**

* Producers push data to a queue. Consumers pop data from the queue.

* Decouples capture timing from logging/UI timing.

**timestamp (`std::chrono::steady_clock::time_point`)**

* Monotonic clock for measuring intervals. Not affected by system time changes.

* Use `steady_clock` for relative timing and alignment between IMUs.

**FIFO burst vs packet**

* The device FIFO stores many fixed-size packets.

* You read a **burst** of `N` bytes. Inside it are `packet_mult` packets. Parse each packet separately.

**Back-computing per-sample time**

* Timestamp once per burst: `t_read`.

* If ODR is `F` Hz and you read `K` packets, you can assign times `t_read - (K-1-i)/F` to packet `i`. This improves alignment.

**Centralized bus locking**

* One mutex in `MCP2221`. Every I²C read/write locks it. Two IMU threads can run safely on one USB handle.

**Redis Streams quick model**

* Append-only log per stream key. Use `XADD imu:<id> * field value ...`.

* Reads with `XREAD`. Ordered by server-generated IDs.

* Pipeline writes: send many commands without waiting, then drain replies in order to reduce round-trips.

**Pipelining**

* Batch multiple `XADD`s before flushing to the socket. Large throughput gains, simpler than transactions.

**Why move fusion out of device code**

* Sensor capture must be real-time and non-blocking.

* Fusion, UI, and logging can stall. Keep them on separate threads to avoid FIFO overflow.

**Two-IMU alignment**

* Both producers timestamp with the same `steady_clock`.

* A consumer can merge by the nearest timestamp or resample to a common grid.

**Zeroing**

* Store `euler_zero_` when the user clicks zero.

* For display only, subtract zero from the fused Euler result. Do not bake it into raw sample data.

**Thread stop pattern**

* Set `run_=false`.

* Join the thread.

* Ensure the queue wakeups cannot block shutdown forever.

**Error handling**

* If I²C read fails, increment a counter, sleep a little, and continue.

* If Redis is down, buffer a small backlog and retry with exponential backoff.

──────────────────────────────────────────────────────────────────────────────

# Quick implementation checklist

1. Add the `include/*.hpp` files as given.

2. Update `mcp2221.cpp` to lock `bus_m_` in every I²C method.

3. In `icm20948.cpp`, keep your register functions and calibration; add `start/stop` and `producer_loop_`. Move your `ReadFIFO()` body into `producer_loop_` and split into `read_fifo_count_`, `read_fifo_burst_`, `parse_fifo_packet_`.

4. Add a stub `redis_logger.cpp`: for now print to console instead of Redis if you prefer.

5. Compile and run with one IMU first. Then enable the second.

6. Only after stability, switch `redis_logger` to actual Redis with hiredis.

This is everything you need offline to implement the `.cpp` bodies behind the headers and understand the terms.